ALTO E

The Alto E is the electric version of our Alto vertical radiator. It saves space due to its vertical design and only needs a socket to quickly achieve a high heat output.

Product:	electrical vertical panel radiator

Pre-installed:	prefilled with heating liquid. RF-receiver mounted including 0,8 m of insulated cable with power plug.

Also supplied:	wall brackets, ETA safety screws and plugs, screws, plugs, installation instructions and RF-thermostat

Lugs:	no lugs

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: Electric power – height – length.

Warranty:	10 years for the radiator and 2 years for the electrical components, as long as the installation instructions have been followed and Henrad’s warranty conditions have been met.

Paint process:	All radiators are degreased, phosphated, primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Colour RF-receiver: 	White for all radiators in Henrad white 9016, A9010, A9003, NT110, NT120, SN110.
Grey for all other colours.

Conformity:	EcoDesign 2018 (EU Regulation 2015/1188)

Technical data:	Voltage 220-240 V ~50 Hz, IPX4, Class II

Type:	11

Height:	1.800 mm

Lengths:	500 | 600 | 700 mm

Output:	1.250 | 1.500 | 1.750 watt

Intelligent RF-thermostat

Temperature setting:	Adjustable from + 10 °C to + 30 °C

4 possible operating modes:	Auto (programming), Comfort, Eco, Frost protection

General usage: 	Choice of 2 application types:

Living room

7-day program: Programming switches between Comfort and Eco. One temperature for Comfort and one for Eco can be set, with a maximum adjustable difference of 5 °C between the two.

Super Comfort function: The temperature is increased by 2 °C, adjustable from 20 to 120 minutes. Manually or via weekly program.

Bathroom

7-day program: Programming switches between Comfort and Boost. The boost can be programmed up to twice a day. Boost: The radiator delivers full power for a maximum of 60 minutes or until a maximum set ambient temperature is reached.

Open window detection:	Automatic switch to frost protection mode when a significant temperature drop is detected.

Child safety:	The control panel can be locked.

Control:	Connection to the radiator is quick and easy. Can control multiple radiators.

Ergonomic: 	backlit display, adjustment is simple and intuitive.