EVEREST PLAN PLINTH D

Low radiators are highly suitable for spaces with large window sections, like shop windows, verandas, winter gardens and under window sills. The cold air next to a window is converted into pleasant warmth.

With a completely flat front and rear, a decorative upper grille and side panels the Everest Plan Plinth D radiator ensure optimal warmth with a minimum of height and a great double finish! Moreover this radiator can be connected either as a valve or as a compact radiator.

Product:	the decorative valve radiator with a flat front and rear and a height of 200 mm

Finishing:	decorative flat front and rear, upper grille and side panels

Pre-installed:	preset Heimeier 4368 or 4369 valve insert

Valve insert:	The integrated adjustable valve insert (without thermostatic head) is factory-fit on the right side of the radiator, CEN certified, tested in accordance with EN215 and compatible with thermostatic heads M30 x 1,5 mm. 
The thermostatic valve insert is factory-set in relation to the size of the radiator. This presetting ensures optimum throughput in the radiator. Factory setting for two-pipe systems, also suitable for one-pipe systems (as long as the valve insert is adjusted to position 8).

Also supplied:	air vent and blind plugs

Connections:	6 x ½” female connection (2 x ½” lateral bottom connection included).

Lugs:	no lugs

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type – height – length.

Warranty:	10 years, as long as the installation instructions have been followed and Henrad’s warranty conditions have been met.

Paint process:	All radiators are degreased, phosphated, primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	110 °C

Conformity:	in accordance with EN442

Types:	22 | 33 | 44

Height:	200 mm

Lengths:	1.000 – 2.600 mm

Depths:	104 | 162 | 220 mm