HYGIENE ECO

Low-temperature panel radiator made from high-quality steel, with serial flow technology (water flows first through the front panel). This results in high energy efficiency, maximum radiation (even in low-temperature heating systems), faster heating rate and less waste of energy.

Product:	the low-temperature valve radiator specifically developed for environments where hygiene and safety are all-important, with serial flow technology, both a centre and lateral bottom connection and a preset valve insert, which can be fitted and the left or right side of the radiator

Finishing:	without upper grille or side panels and without convection fins

Pre-installed:	preset Heimeier 4368 or 4369 valve insert, ECO air vent and blind plugs

Valve insert:	The integrated adjustable valve insert (without thermostatic head) is factory-fit on the right side of the radiator, CEN certified, tested in accordance with EN215 and compatible with thermostatic heads M30 x 1,5 mm. For types 20 and 30 the valve insert can also be fitted on the left side of the radiator, a left version for a type 10 radiator is available on request.

The thermostatic valve insert is factory-set in relation to the size of the radiator. This presetting ensures optimum throughput in the radiator. Factory setting for two-pipe systems, also suitable for one-pipe systems (as long as the valve insert is adjusted to position 8).

Also supplied:	VDI brackets with anti-lift (type Monclac), screws, plugs, air vent, blind plug and installation instructions

Connections:	2 x ¾” male Eurocone centre connection and 2 x ½” female lateral bottom connection. Left version (type 20-30) only with 2 x ½” female lateral bottom connection.

Lugs:	2 pairs of mounting lugs

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type – height – length.

Warranty:	10 years, as long as the installation instructions have been followed and Henrad’s warranty conditions have been met.

Paint process:	All radiators are degreased, phosphated, cataphoretically primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Heat cost allocator:	Suitable for heat cost allocators, both electronic and according to the evaporation principle (covered by the norm of EN834 & EN835).

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	110 °C

Conformity:	in accordance with EN442

Quality assurance:	Hygiene

Types:	10 | 20 | 30

Heights:	400 | 500 | 600 | 700 | 900 mm

Lengths:	400 – 1.600 mm

Depths:	47 | 77 | 158 mm