HYGIENE PLAN

The Hygiene Plan is a panel radiator with a flat front, without convection fins, upper grille and side panels. The point of reference for each project where hygiene and safety are of great importance.

Product:	the decorative naked panel radiator without convection fins

Finishing:	with flat front, without upper grille and side panels

Also supplied:	VDI brackets with anti-lift (type Monclac), screws, plugs, blind plug, air vent and installation instructions

Connections:	4 x ½” female connection

Lugs:	2 pairs of mounting lugs

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type – height – length.

Warranty:	10 years, as long as the installation instructions have been followed and Henrad’s warranty conditions have been met.

Paint process:	All radiators are degreased, phosphated, cataphoretically primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	110 °C

Conformity:	in accordance with EN442

Quality assurance:	NF

Types:	20 | 30

Heights:	400 | 500 | 600 | 700 | 900 mm

Lengths:	400 – 1.600 mm

Depths:	79 | 160 mm