OCEANUS – OCEANUS CT

The Oceanus is a traditional bathroom radiator that retains the timeless character of the bathroom with its simplicity and symmetry of straight, round tubes and D-shaped collectors. This radiator can also be used as a partition wall to create separate areas according to your personal taste. Also available with a centre connection.

Product:	the classic bathroom radiator, also available with a centre connection

Also supplied:	wall brackets, screws, plugs, air vent, blind plugs and installation instructions

Connections:	4 x ½” female connection (6 x ½” if with centre connection)

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type - height – length.

Warranty:	10 years, as long as the installation instructions have been followed, Henrad’s warranty conditions have been met and the radiator is installed outside the splash area.

Paint process:	All radiators are degreased, phosphated and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	95 °C

Conformity:	in accordance with EN442

Heights:	775 | 1.181 | 1.411 | 1.763 | 1.993 mm

Number of tubes:	16 | 24 | 28 | 36 | 40

Lengths:	400 | 450 | 495 | 585 | 737 mm

Tubes:	horizontal, O-section, diameter of 22 mm

Collector:	vertical, D-section