PREMIUM ALL IN

The Premium All In radiator is a fully encased radiator equipped with an integrated adjustable valve insert. Moreover this radiator can be connected either as a valve or as a compact radiator.

This radiator comes with all parts required to mount the radiator.

Product:	the fully encased valve radiator

Finishing:	upper grille and side panels

Pre-installed:	preset Heimeier 4368 or 4369 valve insert, air vent and blind plugs

Valve insert:	The integrated adjustable valve insert (without thermostatic head) is factory-fit on the right side of the radiator, CEN certified, tested in accordance with EN215 and compatible with thermostatic heads M30 x 1,5 mm.

The thermostatic valve insert is factory-set in relation to the size of the radiator. This presetting ensures optimum throughput in the radiator. Factory setting for two-pipe systems, also suitable for one-pipe systems (as long as the valve insert is adjusted to position 8).

Also supplied:	Monclac brackets with anti-lift (VDI compliant when used in combination with included anti-slide clips), screws, plugs and installation instructions

Connections:	6 x ½” female connection (2 x ½” lateral bottom connection included). 
Left handed version: type 21, 22 and 33 are reversible, type 11 is available on request.

Lugs:	Type 11 is equipped with 2 pairs of mounting lugs up to 1.600 mm and 3 pairs from 1.800 mm

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type – height – length.

Warranty:	10 years, as long as the installation instructions have been followed and Henrad’s warranty conditions have been met.

Paint process:	All radiators are degreased, phosphated, cataphoretically primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Heat cost allocator:	Suitable for heat cost allocators, both electronic and according to the evaporation principle (covered by the norm of EN834 & EN835).

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	110 °C

Conformity:	in accordance with EN442

Quality assurance:	NF

Types:	11 | 21 | 22 | 33

Heights:	300 | 400 | 500 | 600 | 700 | 900 mm

Lengths:	400 – 3.000 mm

Depths:	61 | 77 | 100 | 158 mm