PREMIUM MATERNELLE

The Premium Maternelle is an encased valve radiator with a non-water-supplied front plate. It is specially designed for nurseries, kindergartens and retirement homes. The contact surface of the front plate remains at a low temperature at all times, ensuring the safety of everyone from children to adults.

Product:	the fully encased valve radiator with a non-water-supplied front plate

Finishing:	upper grille and side panels

Pre-installed:	preset Heimeier 4368 or 4369 valve insert, air vent and blind plugs

Valve insert:	The integrated adjustable valve insert (without thermostatic head) is factory-fit on the right side of the radiator, CEN certified, tested in accordance with EN215 and compatible with thermostatic heads M30 x 1,5 mm.

The thermostatic valve insert is factory-set in relation to the size of the radiator. This presetting ensures optimum throughput in the radiator. Factory setting for two-pipe systems, also suitable for one-pipe systems (as long as the valve insert is adjusted to position 8).

Also supplied:	VDI brackets, screws, plugs and installation instructions

Connections:	6 x ½” female connection (Left handed version is available on request)

Lugs:	2 pairs of mounting lugs up to 1.600 mm and 3 pairs from 1.800 mm

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type – height – length.

Warranty:	The customer's statutory conformity rights remain fully applicable in accordance with the applicable national law. In addition, Henrad provides a commercial guarantee of 10 years in accordance with Henrad’s applicable guarantee terms and conditions. The current guarantee terms and conditions are available at: www.henrad.eu/en/guarantee-terms.

Paint process:	All radiators are degreased, phosphated, cataphoretically primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	110 °C

Conformity:	in accordance with EN442

Quality assurance:	NF

Type:	32

Heights:	400 | 600 | 700 mm

Lengths:	400 – 2.400 mm

Depth:	158 mm