PREMIUM PLINTH

The Premium Plinth radiator can offer a welcome and attractive solution for locations where you would initially think that a radiator could not be fitted. The cold air close to a window is transformed into cosy warmth.

With its discrete presence this radiator is often used in both private home projects and modern office environments. Moreover this radiator can be connected either as a valve or as a compact radiator.

Product:	the fully encased valve radiator with a height of 200 mm

Finishing:	upper grille and side panels

Pre-installed:	preset Heimeier 4368 or 4369 valve insert

Valve insert:	The integrated adjustable valve insert (without thermostatic head) is factory-fit on the right side of the radiator, CEN certified, tested in accordance with EN215 and compatible with thermostatic heads M30 x 1,5 mm. 
The thermostatic valve insert is factory-set in relation to the size of the radiator. This presetting ensures optimum throughput in the radiator. Factory setting for two-pipe systems, also suitable for one-pipe systems (as long as the valve insert is adjusted to position 8).

Also supplied:	air vent and blind plugs

Connections:	6 x ½” female connection (2 x ½” lateral bottom connection included).

Lugs:	no lugs

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type – height – length.

Warranty:	10 years, as long as the installation instructions have been followed and Henrad’s warranty conditions have been met.

Paint process:	All radiators are degreased, phosphated, primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Heat cost allocator:	Suitable for heat cost allocators, both electronic and according to the evaporation principle (covered by the norm of EN834 & EN835).

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	110 °C

Conformity:	in accordance with EN442

Types:	22 | 33 | 44

Height:	200 mm

Lengths:	1.000 – 2.600 mm

Depths:	100 | 158 | 216 mm