RENO

The Compact radiator with a very specific centre distance to replace old cast-iron and ribbed radiators while using the existing piping system.

Product:	the fully encased compact radiator specifically developed for the renovation market

Finishing:	upper grille and side panels

Also supplied:	installation instructions

Connections:	4 x ½” female connection

Lugs:	2 pairs of mounting lugs up to 1.600 mm and 3 pairs from 1.800 mm

Packaging:	Every radiator is sturdily packaged in high-quality cardboard and wrapped in plastic. The radiator’s characteristics are shown on the label: type – height – length.

Warranty:	10 years, as long as the installation instructions have been followed and Henrad’s warranty conditions have been met.

Paint process:	All radiators are degreased, phosphated, cataphoretically primed and powder-coated in Henrad white 9016 as standard.

Colours:	Henrad white 9016 + 35 different Henrad colours and about 200 RAL-colours are possible

Heat cost allocator:	Suitable for heat cost allocators, both electronic and according to the evaporation principle (covered by the norm of EN834 & EN835).

Max. operating pressure:	10 bar (tested to 13 bar)

Max. operating temperature:	110 °C

Conformity:	in accordance with EN442

Types:	21 | 22 | 33

Heights:	550 | 950 mm

Lengths:	400 – 3.000 mm

Depths:	77 | 100 | 158 mm